Chapter 5

Review Questions

R2. If all the links in the Internet were to provide reliable delivery service, would the TCP reliable delivery service be redundant? Why or why not?

答：不会冗余。因为链路层的可靠传输只保证相邻两个节点之间的正确交付，而 TCP 保证的是端到端可靠传输。即使每一段链路都可靠，数据报仍然可能因为路由器缓存溢出、主机或路由器故障、路径变化等原因丢失或出错，所以仍然需要 TCP 来进行确认、重传、按序交付和端到端差错恢复。

R9. How big is the MAC address space? The IPv4 address space? The IPv6 address space?

答：MAC 地址空间是 2^48，大约为 2.81×10^14；IPv4 地址空间是 2^32，共 4294967296 个地址；IPv6 地址空间是 2^128，大约为 3.4×10^38。

R10. Suppose nodes A, B, and C each attach to the same broadcast LAN (through their adapters). If A sends thousands of IP datagrams to B with each encapsulating frame addressed to the MAC address of B, will C's adapt process these frames? If so, will C's adapter pass the IP datagrams in these frames to the network layer C? How would your answers change if A sends frames with the MAC broadcast address?

答：如果 A 发给 B 的帧目的 MAC 地址是 B 的单播地址，那么 C 的适配器会看到这些帧并检查目的 MAC 地址，但发现不是发给自己的后就会丢弃，不会把其中封装的 IP 数据报交给 C 的网络层。如果 A 发送的是 MAC 广播地址，那么 C 的适配器会接收该帧，并把其中的 IP 数据报上传给网络层；但网络层发现目的 IP 不是自己后，仍会将该数据报丢弃。

R11. Why is an ARP query sent within a broadcast frame? Why is an ARP response sent within a frame with a specific destination MAC address?

答：ARP 请求使用广播帧，是因为发送方只知道对方的 IP 地址，还不知道对方的 MAC 地址，只能在本地局域网里“问所有主机谁拥有这个 IP”。ARP 应答使用单播帧，是因为应答方从请求帧里已经知道了请求方的 MAC 地址，而且只有请求方需要这个映射结果，所以直接单播回复即可，能减少不必要的广播流量。

Problems:

P5. Consider the 7-bit generator, G=10011, and suppose that D has the value 1010101010. What is the value of R?

答：先在数据 D 后面补 4 个 0，得到 10101010100000，再用生成多项式 10011 做模 2 除法，余数为 0100，所以 R = 0100。对应发送比特串为 10101010100100。

P14. Consider three LANs interconnected by two routers, as shown in Figure 5.33.

a. Assign IP addresses to all of the interfaces. For Subnet 1 use addresses of the form 192.168.1.xxx; for Subnet 2 uses addresses of the form

192.168.2.xxx; and for Subnet 3 use addresses of the form 192.168.3.xx

答：下面给出一种可行分配方案。设左边路由器为 R1，右边路由器为 R2。

Subnet 1：R1 左接口 192.168.1.1，A 为 192.168.1.2，B 为 192.168.1.3。

Subnet 2：C 为 192.168.2.1，R1 右接口为 192.168.2.2，R2 左接口为 192.168.2.3，D 为 192.168.2.4。

Subnet 3：R2 右接口为 192.168.3.1，E 为 192.168.3.2，F 为 192.168.3.3。

b. Assign MAC addresses to all of the adapters.

答：下面给出一种可行分配方案。

R1 左接口：00-00-00-00-00-01

A：00-00-00-00-00-02

B：00-00-00-00-00-03

C：00-00-00-00-00-04

R1 右接口：00-00-00-00-00-05

R2 左接口：00-00-00-00-00-06

D：00-00-00-00-00-07

R2 右接口：00-00-00-00-00-08

E：00-00-00-00-00-09

F：00-00-00-00-00-0A

c. Consider sending an IP datagram from Host E to Host B. Suppose all of

the ARP tables are up to date. Enumerate all the steps, as done for the

single-router example in  Section 6.4.1(Link-Layer Addressing and ARP).

答：

1. E 要发送数据报给 B。IP 层发现目的地址 192.168.1.3 不在本子网 192.168.3.0 中，因此先把数据报交给默认网关 R2 的右接口 192.168.3.1。

2. 因为 E 的 ARP 表已经更新，所以它知道 192.168.3.1 对应的 MAC 地址是 00-00-00-00-00-08。

3. E 构造 IP 数据报，源 IP 为 192.168.3.2，目的 IP 为 192.168.1.3；再封装成以太网帧，源 MAC 为 00-00-00-00-00-09，目的 MAC 为 00-00-00-00-00-08，然后发送到 Subnet 3。

4. R2 的右接口收到该帧，发现目的 MAC 是自己，取出其中的 IP 数据报，查路由表后知道应从左接口转发到 Subnet 2，并把下一跳定为 R1 的右接口 192.168.2.2。

5. 因为 R2 的 ARP 表已经更新，所以它知道 192.168.2.2 对应的 MAC 地址是 00-00-00-00-00-05。

6. R2 重新封装该 IP 数据报。新的以太网帧源 MAC 为 00-00-00-00-00-06，目的 MAC 为 00-00-00-00-00-05，然后在 Subnet 2 中发送。

7. R1 的右接口收到该帧，去掉链路层首部和尾部，查看 IP 目的地址 192.168.1.3，发现该地址属于自己直连的 Subnet 1。

8. 因为 R1 的 ARP 表已经更新，所以它知道 B 的 IP 地址 192.168.1.3 对应的 MAC 地址是 00-00-00-00-00-03。

9. R1 再次封装该 IP 数据报。新的以太网帧源 MAC 为 00-00-00-00-00-01，目的 MAC 为 00-00-00-00-00-03，然后在 Subnet 1 中发送。

10. B 的适配器收到该帧后发现目的 MAC 与自己匹配，于是把 IP 数据报交给网络层；网络层发现目的 IP 正是 192.168.1.3，于是交付给上层。

d. Repeat (c), now assuming that the ARP table in the sending host is empty (and the other tables are up to date).

答：

1. E 仍然先判断出目的主机 B 不在本地子网，所以需要先把数据交给默认网关 R2 的右接口 192.168.3.1。

2. 但此时 E 的 ARP 表为空，不知道 192.168.3.1 对应的 MAC 地址，因此 E 先发送一个 ARP 请求广播帧，目的 MAC 为 FF-FF-FF-FF-FF-FF，请求内容是“谁的 IP 是 192.168.3.1，请告诉 192.168.3.2”。

3. Subnet 3 上的所有适配器都会收到这个广播帧，但只有 R2 的右接口发现该 IP 是自己，于是向 E 单播发送 ARP 应答，告诉 E：192.168.3.1 对应的 MAC 地址是 00-00-00-00-00-08。

4. E 收到 ARP 应答后，把 192.168.3.1 和 00-00-00-00-00-08 的映射写入自己的 ARP 表。

5. 然后 E 就按照(c)中的过程发送真正的 IP 数据报：先发给 R2，再由 R2 转发给 R1，最后由 R1 转发给 B。

6. 因为题目说明其他 ARP 表都已经是最新的，所以在后续 R2 到 R1、R1 到 B 的转发过程中，不需要再额外发送 ARP 请求。